📘 Enterprise Network Implementation Report

Daily Summary – Routing, Firewall, DHCP Relay, VoIP & ACL Improvements

🏗️ 1. Network Architecture Overview

Core Devices

- 🟢 Router1 (Primary internal router)

- 🟢 Router2 (Backup internal router)

- 🟢 Firewall (Edge NAT + ACL enforcement)

- 🟢 VoIP Router (CME + VLAN 20 + DHCP relay)

- 🟢 DMZ Server (172.16.10.20 – DHCP / AAA / Services)

- 🟢 ISP / SOHO Router (Upstream)

🌐 2. Subnet & VLAN Design

LAN Subnets (VLSM)

| Subnet | Purpose | Status |
| --- | --- | --- |
| 192.168.1.0/28 | Management | ✅ Working |
| 192.168.1.48/28 | Department | ✅ Working |
| 192.168.1.64/28 | Department | ✅ Working |
| 192.168.1.80/28 | Department | ✅ Working |
| 192.168.1.96/28 | IT Department | ✅ Working |
| 192.168.1.128/27 | Larger LAN | ✅ Working |

📞 Voice VLAN

| Network | Gateway | Status |
| --- | --- | --- |
| 10.50.20.0/24 | 10.50.20.1 | ✅ Working |

🔁 3. HSRP Redundancy

Configured between Router1 & Router2.

interface fa2/0
 ip address 192.168.1.51 255.255.255.240
 standby 1 ip 192.168.1.49
 standby 1 priority 110
 standby 1 preempt

Result

- ✅ Automatic gateway failover

- ✅ Stable redundancy behavior

🔥 4. Firewall & NAT

InterfacesTechnical Explanation – Key Concepts Used in Today’s Network

This section explains why certain commands and IP designs were used, not just what they do.

1️⃣ What Does ip route 0.0.0.0 0.0.0.0 Mean?

Command Example

ip route 0.0.0.0 0.0.0.0 192.168.100.1

What is 0.0.0.0 0.0.0.0?

- 0.0.0.0 = “any network”

- 0.0.0.0 mask = “match everything”

- This is called the default route

It means:

If no specific route matches the destination, send traffic to this next-hop.

Why Use It?

Routers work by matching the most specific route first.

If a router receives traffic for a network it does not know, it must either:

- Drop it

- Or send it somewhere else (default route)

In your design:

| Device | Default Route | Purpose |
| --- | --- | --- |
| Router1 | 0.0.0.0 → 10.1.2.1 | Send unknown traffic to firewall |
| Firewall | 0.0.0.0 → 192.168.100.1 | Send unknown traffic to ISP |

So traffic flow is:

LAN → Router → Firewall → ISP

Without 0.0.0.0, routers would drop unknown traffic immediately.

2️⃣ Why Use 10.50.20.0/24 for Voice Instead of 192.168.1.x?

You intentionally separated voice from data.

Voice Network:

10.50.20.0/24
Gateway: 10.50.20.1

Why Not Use 192.168.1.x?

Because:

✔ Segmentation

Voice traffic is isolated from user PCs.

✔ Broadcast Control

Phones generate signaling traffic (SCCP/SIP, DHCP, TFTP).
Keeping them separate reduces broadcast noise.

✔ QoS Control

In real networks, you can prioritize voice VLAN traffic.

✔ Security

If voice shared 192.168.1.x:

- Lateral movement easier

- Harder to apply policy

So:

Data = 192.168.1.x
Voice = 10.50.20.x

This is proper enterprise design.

3️⃣ Why Use ip helper-address?

On VoIP router:

interface FastEthernet0/0.20
 ip address 10.50.20.1 255.255.255.0
 ip helper-address 172.16.10.20

Phones send DHCP as broadcast:

255.255.255.255

Broadcasts do NOT cross routers.

ip helper-address converts broadcast into unicast and forwards it to:

172.16.10.20 (DHCP server in DMZ)

Without helper:

- Phones never get IP.

4️⃣ NAT – What It Is and Why It Exists

Even though NAT is not fully active in your design, the concept is important.

NAT = Network Address Translation

It changes private IPs into public IPs when exiting to the Internet.

Example:

Internal:

192.168.1.50

Translated to:

192.168.100.2 (firewall outside interface)

Using PAT (overload):

ip nat inside source list NAT_INSIDE interface FastEthernet1/0 overload

What This Means

- Multiple internal devices share one public IP.

- Translation happens only when exiting ip nat outside.

In Your Current Setup

You routed everything manually:

Router1 → Firewall → ISP

But NAT is not fully enforced.

Right now:

- Internet works because routing exists.

- Not because NAT is properly built.

If ISP were real:

- Private IPs would not work.

- NAT would be mandatory.

5️⃣ Firewall Routing Design Explained

Firewall Interfaces

FastEthernet0/0   172.16.10.1/24   (DMZ)      ip nat inside
FastEthernet7/0   10.1.1.1/30      (Router1)  ip nat inside
FastEthernet8/0   10.1.2.1/30      (Router2)  ip nat inside
FastEthernet1/0   192.168.100.2/30 (ISP)      ip nat outside

Firewall Routes

Default:

ip route 0.0.0.0 0.0.0.0 192.168.100.1

Internal routes:

ip route 192.168.1.0 255.255.255.0 10.1.1.2
ip route 10.50.20.0 255.255.255.0 10.1.1.2
ip route 10.50.20.0 255.255.255.0 10.1.2.2 10

Why These Exist

Firewall must know:

- Where LAN networks live

- Where voice network lives

Otherwise default route sends internal traffic to ISP.

That was the earlier problem.

6️⃣ Router1 & Router2 Connections Explained

Router1

Interfaces:

10.1.1.2/30 → Firewall
10.255.2.1/30 → VoIP Router
192.168.1.x → LAN segments

Route added:

ip route 10.50.20.0 255.255.255.0 10.255.2.2

Meaning:
Voice network lives behind VoIP router.

Router2

Interfaces:

10.1.2.2/30 → Firewall
10.255.2.5/30 → VoIP Router

Route added:

ip route 10.50.20.0 255.255.255.0 10.255.2.6

7️⃣ Why Return Routes Matter

Routing must be symmetric.

Example:

Phone → DMZ server
Path:

Phone → VoIP → Router1 → Firewall → DMZ

Return must follow:

DMZ → Firewall → Router1 → VoIP → Phone

If Router1 lacks route to 10.50.20.0/24:

- It forwards to default route.

- Firewall may send to ISP.

- Packet drops.

That was one of today’s major fixes.

8️⃣ Why Transit Networks Use /30

Example:

10.1.1.0/30

Mask:

255.255.255.252

Provides:

- 2 usable IP addresses

- Perfect for point-to-point links

- Reduces broadcast domain

- Prevents overlapping confusion

Used for:

- Router ↔ Firewall

- Router ↔ VoIP

9️⃣ Why IT ACL Works the Way It Does

You applied ACL outbound from IT switch.

Effect:

- External device sends ping.

- IT device receives it.

- IT reply tries to leave subnet.

- ACL blocks it.

This is intentional containment.

Benefits:

- IT sees the attempt.

- Lateral movement prevented.

- Monitoring preserved.

- Traffic not silently filtered upstream.

This is closer to real security design than simply blocking at firewall.

10️⃣ Current Design Summary

Routing-based architecture:

LAN → Router1/Router2 → Firewall → ISP
Voice → VoIP Router → Router1/2 → Firewall → DMZ

NAT:

- Partially configured

- Not fully required in PT simulation

- Would be mandatory in real ISP scenario

All routing:

- Static

- Deterministic

- Symmetric

- No loops

interface FastEthernet0/0
 ip address 172.16.10.1 255.255.255.0
 ip nat inside

interface FastEthernet1/0
 ip address 192.168.100.2 255.255.255.252
 ip nat outside

PAT

ip nat inside source list NAT_INSIDE interface FastEthernet1/0 overload

Status

- ✅ Internet traffic NATed correctly

- ❌ Initially internal traffic misrouted to ISP (fixed by adding static routes)

📞 5. VoIP & DHCP Relay

interface FastEthernet0/0.20
 encapsulation dot1Q 20
 ip address 10.50.20.1 255.255.255.0
 ip helper-address 172.16.10.20

Status

- ✅ DHCP from DMZ server working

- ❌ Initially failed due to asymmetric routing

- ✅ Fixed by adding proper return routes

🛡️ 6. IT Department ACL – Final Design & Improvement

ACL applied on SW6 (IT switch) outbound toward routers.

ip access-list extended IT_POLICY
 permit ip 192.168.1.96 0.0.0.15 192.168.1.0 0.0.0.15
 permit ip 192.168.1.96 0.0.0.15 172.16.10.0 0.0.0.255
 deny   ip 192.168.1.96 0.0.0.15 10.0.0.0 0.255.255.255
 deny   ip 192.168.1.96 0.0.0.15 172.16.0.0 0.15.255.255
 deny   ip 192.168.1.96 0.0.0.15 192.168.0.0 0.0.255.255
 permit ip 192.168.1.96 0.0.0.15 any

Applied on routed interfaces:

interface FastEthernet0/5
 ip access-group IT_POLICY out

🔎 Important Behavior Observed

Traffic behavior:

- Packet enters IT VLAN.

- Packet reaches IT device.

- When response attempts to leave subnet → ACL blocks it.

- Traffic dropped at switch.

This is intentional and GOOD ✅

Why?

- 🟢 The IT devices can see that someone attempted communication.

- 🟢 Traffic is logged/monitored.

- 🟢 Unauthorized communication does NOT leave the subnet.

- 🟢 Attack attempts become visible internally.

- 🟢 Outbound control is enforced at the source boundary.

This creates a controlled “containment” model instead of silent blocking upstream.

For a security-focused IT department, this is desirable:

- Monitor attempts

- Prevent lateral movement

- Maintain logging visibility

❌ 7. Issues Fixed Today

1️⃣ Wrong DMZ Server IP

- ❌ Server misaddressed

- 🔁 Changed to correct 172.16.10.20

- ✅ Immediately restored service reachability

2️⃣ VoIP Router Mis-Cabling

- ❌ Router1 connected to wrong interface (Fa0/0)

- ❌ Duplicate link detected via CDP

- 🔁 Removed duplicate cable

- 🔁 Used correct L3 interface only

- ✅ ARP started resolving properly

3️⃣ Overlapping Transit Networks

- ❌ Same /30 used on two interfaces

- IOS error: “overlaps”

- 🔁 Created separate /30 networks:

  - 10.255.2.0/30 (Router1)

  - 10.255.2.4/30 (Router2)

- ✅ Clean routing

4️⃣ Missing Return Routes

- ❌ Router2 lacked route to voice network

- 🔁 Added static route

- ✅ Restored symmetry

5️⃣ Firewall Sending Internal Traffic to ISP

- ❌ Default route catching internal traffic

- 🔁 Added static route to 10.50.20.0/24

- ✅ Internal traffic no longer exits to ISP

🧠 Commands Mastered

Routing

show ip route
ip route <network> <mask> <next-hop>

ARP

show arp

CDP

show cdp neighbors detail

ACL

show access-lists
ip access-group <acl> in|out

NAT

show ip nat translations

📊 Final Network State

| Feature | Status |
| --- | --- |
| HSRP Redundancy | ✅ Stable |
| Static Routing | ✅ Symmetric |
| Firewall NAT | ✅ Controlled |
| VoIP DHCP | ✅ Working |
| DMZ Services | ✅ Reachable |
| IT ACL Containment | ✅ Enforced |
| Routing Loops | ❌ Eliminated |
| Overlapping Subnets | ❌ Eliminated |

🎯 Key Takeaways for the Team

✔️ Every route needs a return path
✔️ Transit networks must be isolated
✔️ CDP helps detect physical mistakes
✔️ Default routes can misroute internal traffic
✔️ ACL placement changes traffic visibility
✔️ Controlled drop at the edge of a subnet improves monitoring

Technical Explanation – Key Concepts Used in Today’s Network

This section explains why certain commands and IP designs were used, not just what they do.

1️⃣ What Does ip route 0.0.0.0 0.0.0.0 Mean?

Command Example

ip route 0.0.0.0 0.0.0.0 192.168.100.1

What is 0.0.0.0 0.0.0.0?

- 0.0.0.0 = “any network”

- 0.0.0.0 mask = “match everything”

- This is called the default route

It means:

If no specific route matches the destination, send traffic to this next-hop.

Why Use It?

Routers work by matching the most specific route first.

If a router receives traffic for a network it does not know, it must either:

- Drop it

- Or send it somewhere else (default route)

In your design:

| Device | Default Route | Purpose |
| --- | --- | --- |
| Router1 | 0.0.0.0 → 10.1.2.1 | Send unknown traffic to firewall |
| Firewall | 0.0.0.0 → 192.168.100.1 | Send unknown traffic to ISP |

So traffic flow is:

LAN → Router → Firewall → ISP

Without 0.0.0.0, routers would drop unknown traffic immediately.

2️⃣ Why Use 10.50.20.0/24 for Voice Instead of 192.168.1.x?

You intentionally separated voice from data.

Voice Network:

10.50.20.0/24
Gateway: 10.50.20.1

Why Not Use 192.168.1.x?

Because:

✔ Segmentation

Voice traffic is isolated from user PCs.

✔ Broadcast Control

Phones generate signaling traffic (SCCP/SIP, DHCP, TFTP).
Keeping them separate reduces broadcast noise.

✔ QoS Control

In real networks, you can prioritize voice VLAN traffic.

✔ Security

If voice shared 192.168.1.x:

- Lateral movement easier

- Harder to apply policy

So:

Data = 192.168.1.x
Voice = 10.50.20.x

This is proper enterprise design.

3️⃣ Why Use ip helper-address?

On VoIP router:

interface FastEthernet0/0.20
 ip address 10.50.20.1 255.255.255.0
 ip helper-address 172.16.10.20

Phones send DHCP as broadcast:

255.255.255.255

Broadcasts do NOT cross routers.

ip helper-address converts broadcast into unicast and forwards it to:

172.16.10.20 (DHCP server in DMZ)

Without helper:

- Phones never get IP.

4️⃣ NAT – What It Is and Why It Exists

Even though NAT is not fully active in your design, the concept is important.

NAT = Network Address Translation

It changes private IPs into public IPs when exiting to the Internet.

Example:

Internal:

192.168.1.50

Translated to:

192.168.100.2 (firewall outside interface)

Using PAT (overload):

ip nat inside source list NAT_INSIDE interface FastEthernet1/0 overload

What This Means

- Multiple internal devices share one public IP.

- Translation happens only when exiting ip nat outside.

In Your Current Setup

You routed everything manually:

Router1 → Firewall → ISP

But NAT is not fully enforced.

Right now:

- Internet works because routing exists.

- Not because NAT is properly built.

If ISP were real:

- Private IPs would not work.

- NAT would be mandatory.

5️⃣ Firewall Routing Design Explained

Firewall Interfaces

FastEthernet0/0   172.16.10.1/24   (DMZ)      ip nat inside
FastEthernet7/0   10.1.1.1/30      (Router1)  ip nat inside
FastEthernet8/0   10.1.2.1/30      (Router2)  ip nat inside
FastEthernet1/0   192.168.100.2/30 (ISP)      ip nat outside

Firewall Routes

Default:

ip route 0.0.0.0 0.0.0.0 192.168.100.1

Internal routes:

ip route 192.168.1.0 255.255.255.0 10.1.1.2
ip route 10.50.20.0 255.255.255.0 10.1.1.2
ip route 10.50.20.0 255.255.255.0 10.1.2.2 10

Why These Exist

Firewall must know:

- Where LAN networks live

- Where voice network lives

Otherwise default route sends internal traffic to ISP.

That was the earlier problem.

6️⃣ Router1 & Router2 Connections Explained

Router1

Interfaces:

10.1.1.2/30 → Firewall
10.255.2.1/30 → VoIP Router
192.168.1.x → LAN segments

Route added:

ip route 10.50.20.0 255.255.255.0 10.255.2.2

Meaning:
Voice network lives behind VoIP router.

Router2

Interfaces:

10.1.2.2/30 → Firewall
10.255.2.5/30 → VoIP Router

Route added:

ip route 10.50.20.0 255.255.255.0 10.255.2.6

7️⃣ Why Return Routes Matter

Routing must be symmetric.

Example:

Phone → DMZ server
Path:

Phone → VoIP → Router1 → Firewall → DMZ

Return must follow:

DMZ → Firewall → Router1 → VoIP → Phone

If Router1 lacks route to 10.50.20.0/24:

- It forwards to default route.

- Firewall may send to ISP.

- Packet drops.

That was one of today’s major fixes.

8️⃣ Why Transit Networks Use /30

Example:

10.1.1.0/30

Mask:

255.255.255.252

Provides:

- 2 usable IP addresses

- Perfect for point-to-point links

- Reduces broadcast domain

- Prevents overlapping confusion

Used for:

- Router ↔ Firewall

- Router ↔ VoIP

9️⃣ Why IT ACL Works the Way It Does

You applied ACL outbound from IT switch.

Effect:

- External device sends ping.

- IT device receives it.

- IT reply tries to leave subnet.

- ACL blocks it.

This is intentional containment.

Benefits:

- IT sees the attempt.

- Lateral movement prevented.

- Monitoring preserved.

- Traffic not silently filtered upstream.

This is closer to real security design than simply blocking at firewall.

10️⃣ Current Design Summary

Routing-based architecture:

LAN → Router1/Router2 → Firewall → ISP
Voice → VoIP Router → Router1/2 → Firewall → DMZ

NAT:

- Partially configured

- Not fully required in PT simulation

- Would be mandatory in real ISP scenario

All routing:

- Static

- Deterministic

- Symmetric

- No loops