# Enterprise Network Design & Implementation Summary

# 1. Network Architecture Overview

• Router1 (R1) and Router2 (R2) configured for gateway redundancy using HSRP.
• Firewall deployed as edge device performing NAT (PAT overload).
• SOHO/ISP router used as upstream gateway.
• Multiple VLSM-based LAN segments implemented.

# 2. Subnet Design (VLSM)

Implemented Subnets:
• 192.168.1.0/28   (Gateway VIP: 192.168.1.1)
• 192.168.1.48/28  (Gateway VIP: 192.168.1.49)
• 192.168.1.64/28  (Gateway VIP: 192.168.1.65)
• 192.168.1.80/28  (Gateway VIP: 192.168.1.81)
• 192.168.1.96/28  (Gateway VIP: 192.168.1.97)
• 192.168.1.128/27 (Gateway VIP: 192.168.1.129)

# 3. HSRP Redundancy Implementation

HSRP configured on all LAN interfaces between R1 and R2.

Key Commands Used:
standby <group> ip <virtual-ip>
standby <group> priority <value>
standby <group> preempt
show standby brief

Result: Automatic gateway failover without manual routing changes.

# 4. Firewall Configuration & NAT

Inside Interface: 172.16.10.1/24 (ip nat inside)
Outside Interface: 192.168.100.2/30 (ip nat outside)

PAT Configuration:
ip nat inside source list NAT_INSIDE interface FastEthernet1/0 overload

Verification Commands:
show ip nat translations
show ip nat statistics

# 5. Static Routing Design

Default Route (Firewall → ISP):
ip route 0.0.0.0 0.0.0.0 192.168.100.1

Internal Routes (Firewall → Internal Router):
ip route 192.168.1.0 255.255.255.0 10.1.1.2

Routing Verification:
show ip route
show ip route static

# 6. ACL Implementation

Extended ACL 100 applied inbound on firewall interfaces.

Example:
access-list 100 permit ip 192.168.1.0 0.0.0.255 any
access-list 100 deny ip any any

Applied using:
ip access-group 100 in

# 7. Troubleshooting & Diagnostics

Key Diagnostic Commands Used:
show ip interface brief
show arp
ping
traceroute
show run | include ip route

Identified and resolved:
• Incorrect HSRP virtual IP assignments
• Routing loops between firewall and SOHO router
• Inactive static routes due to unreachable next-hop
• NAT translating internal-to-internal traffic
• Interface up/down link issues

# 8. Final Network Behavior

• Automatic gateway failover via HSRP.
• Internal traffic routed correctly without NAT.
• External traffic NATed at firewall only.
• ISP return routing properly configured.
• No routing loops or asymmetric routing.

3. Redundancy Implementation

HSRP (Hot Standby Router Protocol)

Implemented between Router1 and Router2 for gateway redundancy.

Example Configuration (Router1)

interface fa2/0
 ip address 192.168.1.51 255.255.255.240
 standby 1 ip 192.168.1.49
 standby 1 priority 110
 standby 1 preempt

Key Commands Used

show standby brief
standby <group> ip <virtual-ip>
standby <group> priority <value>
standby <group> preempt

Purpose:

- Provides automatic gateway failover

- Prevents manual route changes during router failure

4. Firewall Configuration

Inside / Outside Interfaces

interface FastEthernet0/0
 ip address 172.16.10.1 255.255.255.0
 ip nat inside

interface FastEthernet1/0
 ip address 192.168.100.2 255.255.255.252
 ip nat outside

5. NAT Configuration

PAT (Overload)

ip nat inside source list NAT_INSIDE interface FastEthernet1/0 overload

NAT Access List

ip access-list standard NAT_INSIDE
 permit 192.168.1.0 0.0.0.255
 permit 192.168.2.0 0.0.0.255
 permit 172.16.10.0 0.0.0.255

Verification Commands

show ip nat translations
show ip nat statistics

Purpose:

- Allow internal subnets to access ISP

- Perform address translation only when exiting outside interface

6. Static Routing

Default Route (Firewall → ISP)

ip route 0.0.0.0 0.0.0.0 192.168.100.1

Internal Routes (Firewall → Router1 or Router2)

ip route 192.168.1.0 255.255.255.0 10.1.1.2

7. ACL Implementation

Used to control traffic entering firewall.

access-list 100 permit ip 10.1.1.0 0.0.0.3 any
access-list 100 permit ip 192.168.1.0 0.0.0.255 any
access-list 100 deny ip any any

Applied to interface:

interface fa7/0
 ip access-group 100 in

8. Troubleshooting Techniques Used

Routing

show ip route
show ip route static
show run | include ^ip route

Interface Status

show ip interface brief
show interface <interface>

ARP

show arp

Connectivity

ping <ip>
traceroute <ip>